SVKM's NMIMS · Mukesh Patel School of Technology Management & Engineering, Mumbai

Modern Day Robotics & Its Industrial Applications (MDRIIA - 702CO0E012) · B.Tech Computer Science & Business Systems (CSBS), in partnership with TCS · AY 2026–27 Odd Semester

GROUP 09 TECHNICAL DOSSIER

Capstone ICA (20 marks): CDIO Robotics Engineering & MuJoCo Physics 10 · CSBS Techno-Economic Business Systems Analysis 5 · Studio Showcase & Viva Voce 5

| Group · project | Group 09 · Autonomous Ground Vehicle (UGV) with LiDAR Perception for Hazardous Terrain Reconnaissance |
| --- | --- |
| Monorepo folder | https://github.com/sunny-nanade/NMIMS-MPSTME-MDRIIA-2026/tree/main/Group_09_Arnav_Vihan_Pratik |
| Submit (3 places) | (1) PDF on MS Teams, turned in by EVERY member · (2) this filled Word file AND its PDF committed to Group_09_Arnav_Vihan_Pratik/docs/MDRIIA_Group_09_Technical_Dossier.docx / .pdf · (3) LinkedIn video URL in Section 5. Due Monday, 26-Oct-2026, 23:59 IST. Studio Showcase, Expo & Viva Voce: Saturday, 31-Oct-2026. |
| No slides | There is no PPT for MDRIIA. At the Expo and in your video you present the live MuJoCo simulation, your business sensitivity graphs, and a webcam picture-in-picture of every member. |
| How to fill this file | Use Microsoft Word (desktop). Choose answers from the 'Select ▾' drop-downs and type into the 'type here' fields. Do not delete or retype them and do not edit this file in Google Docs: the fields are how your answers are read. |

| Student name | Roll No. | SAP ID | Capstone role(s) | Specialisation (finalised titles sheet / monorepo) |
| --- | --- | --- | --- | --- |
| Arnav Saurabh Surve | E064 | 70362400067 | Lead Robotics Systems Architect | Lead UGV Skid-Steer Dynamics & MuJoCo Terrain Modeler |
| Vihan Shripad Joshi | E070 | 70362400086 | Autonomous Navigation / Vision Specialist + Telemetry QA Lead | LiDAR Perception, 3D Elevation Mapping & Obstacle Segmentation Lead |
| Pratik Mangesh Gaikwad | E073 | 70362300032 | CSBS Business & Techno-economic Systems Analyst | CSBS Hazardous Operations Safety & Teleoperation Latency Analyst |

Roll numbers and SAP IDs are from the official student list. Tell Dr. Nanade by Tuesday, 20-Oct-2026 if any name, ID or role is wrong. Groups with fewer than four members combine roles as shown.

SECTION 1 · Problem Bottleneck, Industry Standards & Aalborg Research Title    Component 1 · CDIO Robotics (10)

| Authorised Aalborg title | How can an autonomous ground vehicle utilizing simulated LiDAR rangefinders and traversability cost-mapping in MuJoCo navigate unknown unstructured hazardous terrain while reducing teleoperation cognitive workload and communication latency? |
| --- | --- |
| Original draft topic | UGV for defence and hazardous applications |
| Why the title changed | The student submission addressed a critical defence/industrial robotics challenge: hazardous terrain reconnaissance. However, 'map unknown terrain and autonomously navigate to designated points' is an overly broad textbook definition of general SLAM. It missed the unique physical constraint of hazardous unstructured environments: traversability analysis (determining whether rubble, rocks, or slopes can be driven over or will high-center the chassis). |

1.1 Baseline friction: claim verification table

Each statement was carried over from your titles sheet or your monorepo scripts. Before you use it anywhere (dossier, video, paper), find the primary source yourself and record it, or correct or delete the claim. Statistics: cite the original study, not a blog or an AI answer.

| Claim (as carried over) | Faculty pre-audit note | Primary source you verified (author, year, venue, DOI/URL) | Source type | Keep / Correct / Delete |
| --- | --- | --- | --- | --- |
| Autonomous local navigation reduces RF telemetry bandwidth requirements by 80%. | No traceable source. Measure the data rate in your own simulation or delete. | type here | Select ▾ | Select ▾ |
| Operator NASA-TLX cognitive load falls from 78 to 32. | A human-subject result. It cannot come from a MuJoCo study; delete unless you test real operators. | type here | Select ▾ | Select ▾ |
| Teleoperation latency of 350 ms. | A design parameter in your code. Cite a source for typical teleoperation latencies or label it as an assumption. | type here | Select ▾ | Select ▾ |

1.2 Governing industry standards

Decide for each standard whether it governs your system, and name the clause or requirement you design against (cite number, part and year). 'No' is a correct answer when you can say why. The hint is the faculty view; you may disagree with a reason.

| Standard | Scope | Faculty hint for your project | Applies? | Clause / requirement you use, or why not |
| --- | --- | --- | --- | --- |
| ISO 10218-1/-2 | Safety requirements for industrial robots (robot, and robot integration) | No | Select ▾ | type here |
| ISO 3691-4 | Driverless industrial trucks / AMRs and their systems: safety requirements | Partly: driverless vehicle safety concepts (personnel detection, speed limits) | Select ▾ | type here |
| ANSI/RIA R15.06 | US national adoption of ISO 10218 (industrial robot safety) | No | Select ▾ | type here |
| IEC 60601-1 | Medical electrical equipment: basic safety and essential performance | No | Select ▾ | type here |
| Domain-specific | No single standard covers defence/hazard UGVs: state the safety assumptions you use | Recommended: check it | Select ▾ | type here |

1.3 Documented baseline friction (who struggles, with what, how it is measured today)

| Baseline metric and its value today (with unit) | type here |
| --- | --- |
| Source of the baseline value (author, year, DOI/URL) | type here |
| Who carries the cost today (role / stakeholder) | type here |

SECTION 2 · MuJoCo Rigid-Body Physics Modelling & Python Closed-Loop Control    Component 1 · CDIO Robotics (10)

| MuJoCo mechanism (titles sheet) | MuJoCo MJCF Rigid-Body Model with Autonomous Ground Vehicle (UGV) with LiDAR Perception for Hazardous Terrain Reconnaissance Physics & Closed-Loop Control |
| --- | --- |
| MJCF model | Group_09_Arnav_Vihan_Pratik/models/hazardous_terrain_ugv.xml |
| Controller | Group_09_Arnav_Vihan_Pratik/src/rough_terrain_recon_controller.py |
| Contact & solver settings in the model today | timestep 0.002 s (500 Hz) · wheel friction 0.85–0.9 / 0.01 / 0.001 · solref 0.01 1 · solimp 0.9 0.95 0.001 · 4 wheel motors + 1 |
| Core state machine (FSM) | OPERATOR_TRACKING ↔ AUTONOMOUS_OVERRIDE (obstacle / tip-over risk) → TRAVERSE → GOAL; 350 ms shared-autonomy latency buffer |
| Equations of motion & control | Newton-Euler (rigid body): m·v̇ = ΣF, I·ω̇ + ω × (I·ω) = Στ; multibody form M(q)·q̈ + C(q, q̇)·q̇ + g(q) = τ + Jᵀ·f_ext Skid-steer: ω_L = (v − ω·B/2) / r, ω_R = (v + ω·B/2) / r (B = 0.64 m, r = 0.14 m) Slip ratio s = (ω·r − v) / (ω·r); traversability cost c = w₁·slope + w₂·roughness + w₃·step height PID tracking: u(t) = K_p·e(t) + K_i·∫e dt + K_d·ė(t) Damped least-squares IK (manipulators): Δq = Jᵀ·(J·Jᵀ + λ²·I)⁻¹·Δx |

Faculty code audit: Your ROI script hard-codes NASA-TLX 78.4 → 28.2. NASA-TLX is a human questionnaire: you cannot obtain it from simulation. Delete it or run a real operator study.

2.1 Model and controller settings you actually used (from your final code)

| MJCF timestep (s) in the final model | type here |
| --- | --- |
| Main contact friction (sliding / torsional / rolling) | type here |
| solref (timeconst dampratio) and why you chose it | type here |
| Controller law used | Select ▾ |
| Final gains (e.g. Kp, Ki, Kd or λ) with units | type here |
| FSM states implemented in code (list) | type here |
| Model loads and runs: python -c "import mujoco; mujoco.MjModel.from_xml_path(...)" | Select ▾ |

2.2 Authorship of each script

Several scripts were provided as starter scaffolds. Mark honestly what you wrote, changed, or used as given. Viva questions target the code you claim.

| File | Written / Modified / Used as given | Main author (Roll) | What you changed or added (1 line) |
| --- | --- | --- | --- |
| models/hazardous_terrain_ugv.xml | Select ▾ | Select ▾ | type here |
| src/rough_terrain_recon_controller.py | Select ▾ | Select ▾ | type here |
| analytics/hazardous_recon_economics.py | Select ▾ | Select ▾ | type here |
| type here | Select ▾ | Select ▾ | type here |
| type here | Select ▾ | Select ▾ | type here |

2.3 Key code snippet (≤ 25 lines) implementing your FSM or control law:

[ Paste code as text (Consolas 8 pt), not a screenshot. Include the file name and line numbers. ]

2.4 Free-body diagram and one equation worked with your own numbers and units:

[ FBD (phone photo of a hand sketch is fine) → symbols explained → worked example → where this value appears in your telemetry CSV ]

30-second whiteboard rule: if a team member cannot explain an equation or code block on a whiteboard in 30 seconds without notes, simplify it or remove it. It will be asked in the viva.

SECTION 3 · CSBS Techno-Economic Business Systems Modelling    Component 2 · CSBS Techno-Economic (5)

| Business script | Group_09_Arnav_Vihan_Pratik/analytics/hazardous_recon_economics.py (rename or add *_roi.py if you restructure it) |
| --- | --- |
| CSBS business case (titles sheet) | CSBS Defence & Industrial Risk Economics: Industrial boiler inspection, collapsed building search, and hazardous chemical spill mapping incur severe life-safety risks and liability costs. Deploying an autonomous UGV eliminates the exposure of human response personnel to dangerous environmental hazards. Compared to pure teleoperation, autonomous local navigation reduces RF telemetry bandwidth requirements by 80%, permitting robust operation over degraded satellite/mesh links while cutting operator mission fatigue (NASA-TLX cognitive load reduced from 78 to 32). |
| Constants hard-coded in the script today | annual_confined_space_entries_baseline = 48.0, tlx_manual_teleop = 78.4, tlx_shared_autonomy = 28.2, t_hazmat_prep_and_entry_hours = 6.5, k_capex_ugv = 0.30 → each one needs a source in 3.1 or must be computed from your simulation. |
| Owner | CSBS Business & Techno-economic Systems Analyst: E073. Tags in this section carry that roll number. |

Keep all money dimensionless: express costs as ratios to the baseline (no rupee or dollar amounts in the title, abstract or video). Every input needs a source or must be a clearly labelled assumption that you vary in the sensitivity graph.

3.1 Business model: formulas, your inputs and your results

| Quantity | Formula (define every symbol in your script) | Inputs used, with source or 'assumption' | Result (with unit) |
| --- | --- | --- | --- |
| Labour reallocation H_reclaimed | H = responder hours per year no longer spent inside hazardous zones (person-hours/year) | type here | type here |
| Payback horizon P_payback (dimensionless) | P_payback = CapEx / (T_life · ΔOpEx_annual); P < 1 ⇒ pays back within life T_life. Also report T_payback = CapEx / ΔOpEx in months. | type here | type here |
| Normalised Cost Reduction Index NCRI | NCRI = (C_baseline − C_robot) / C_baseline (0–1); cost-parity ratio κ = C_robot / C_baseline = 1 − NCRI | type here | type here |
| Queuing delay | M/M/1: inspection requests (λ per week) served by one UGV (μ per week). ρ = λ / (c·μ) < 1; M/M/1: W_q = ρ / (μ − λ); M/M/c: W_q = C(c, λ/μ) / (c·μ − λ) (Erlang C) | type here | type here |
| Sensitivity analysis | Vary the most uncertain input ±30 % (or across your scenario range) and plot P_payback and NCRI; report the break-even value. | type here | type here |

| Queuing model used | Select ▾ |
| --- | --- |
| λ, μ, c values used (with units) | type here |
| Payback in months (T_payback) | type here |
| Script output reproduces every number above when re-run | Select ▾ |

3.2 FMEA: failure modes, effects and risk priority (RPN = S × O × D, each 1–10)

Two failure modes are seeded from your project; add at least two more. Score before and after your mitigation (design change, control limit, sensor, procedure). The post-mitigation RPN must be justified by something you can show in the simulation or the code.

| Failure mode | Effect on mission / business | S | O | D | RPN pre | Mitigation (what you changed) | RPN post | Shown in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High-centring / tip-over on rubble step | type here | Select ▾ | Select ▾ | Select ▾ | type here | type here | type here | Select ▾ |
| Communication delay → late stop near hazard | type here | Select ▾ | Select ▾ | Select ▾ | type here | type here | type here | Select ▾ |
| type here | type here | Select ▾ | Select ▾ | Select ▾ | type here | type here | type here | Select ▾ |
| type here | type here | Select ▾ | Select ▾ | Select ▾ | type here | type here | type here | Select ▾ |
| type here | type here | Select ▾ | Select ▾ | Select ▾ | type here | type here | type here | Select ▾ |

| [ Sensitivity graph: P_payback / NCRI vs the uncertain input ] | [ Queuing graph: W_q vs utilisation ρ, or labour-reallocation chart ] |
| --- | --- |

SECTION 4 · Simulation Telemetry, Monte Carlo Testing & Empirical Validation    Component 1 · CDIO Robotics (10)

| Evaluation protocol (titles sheet) | Run N = 30 trials across 3 roughness index rubble courses. Measure: mean speed (m/s), wheel slip ratio, chassis roll/pitch pitch stability margin (degrees), obstacle avoidance success rate (%), and human operator intervention frequency. |
| --- | --- |
| Telemetry standard | Log at 500 Hz (MJCF timestep 0.002 s, or every 2nd step at 0.001 s): time, state (q, q̇ or pose, velocity), reference, tracking error, actuator torques/forces, FSM state, events. One CSV per run or one CSV with a run_id column. |
| Monte Carlo | N ≥ 30 runs per condition (more if your protocol says so). Randomise disturbances and initial conditions from stated distributions; record every random seed. |
| Required measures | Tracking RMSE = √(Σ‖p − p_ref‖² / n) on the planned waypoint path vs MuJoCo chassis position; control effort E = ∫τ² dt (sum of τ²·Δt over all actuators); your primary task metric from the protocol. |
| Statistics | Compare the proposed controller with a baseline condition: Student's / Welch's t-test (two-sided, α = 0.05), Cohen's d = (x̄₁ − x̄₂) / s_pooled, and a 95 % confidence interval. |
| Where real trial data goes | Group_09_Arnav_Vihan_Pratik/analytics/*_real_trials.csv, produced by your controller. Files named SIMULATED_* are starter targets, never results. |
| Owner | Telemetry QA Lead: E070. |

| Baseline condition you compare against | type here |
| --- | --- |
| Monte Carlo runs per condition (N) | type here |
| Randomised parameters and their distributions | type here |
| Logging rate actually achieved (Hz) | type here |
| Random seeds recorded for every run | Select ▾ |
| Real-trials CSV path(s) in the repo | type here |

| Measure | N | Baseline mean ± SD | Proposed mean ± SD | Unit | Better? (direction) |
| --- | --- | --- | --- | --- | --- |
| Tracking RMSE | type here | type here | type here | type here | Select ▾ |
| Control effort ∫τ² dt | type here | type here | type here | type here | Select ▾ |
| type here | type here | type here | type here | type here | Select ▾ |

| Test used | Select ▾ |
| --- | --- |
| t statistic, degrees of freedom, p-value (primary metric) | type here |
| Cohen's d (primary metric) | type here |
| 95 % confidence interval of the difference | type here |

| [ Trajectory plot: reference vs actual, one representative run ] | [ Box-plot of the primary metric: baseline vs proposed, N runs each ] |
| --- | --- |

DATA PROVENANCE DECLARATION (mandatory). Reporting generated target numbers as experimental results is research misconduct and scores zero for this component.

| Every number in Section 4 comes from MuJoCo runs of OUR controller, logged by our code | Select ▾ |
| --- | --- |
| No value comes from SIMULATED_* files, generate_paper_figures.py or any np.random 'target' data | Select ▾ |
| Our plots re-generate from the *_real_trials.csv files in the repo | Select ▾ |
| Every input in Section 3 has a source or is labelled as an assumption | Select ▾ |

SECTION 5 · LinkedIn Video Showcase, AI Verification Audit & Peer Review    Component 3 · Showcase & Viva (5)

| LinkedIn post URL | type here |
| --- | --- |
| Video length (mm:ss) | type here |
| Video shows the live MuJoCo simulation (not slides) | Select ▾ |
| Video shows our business sensitivity graph | Select ▾ |
| Webcam picture-in-picture: every member appears and speaks | Select ▾ |

5.1 AI Interaction Audit Log: three cases where AI gave wrong robotics or economic data

Examples: a wrong MuJoCo attribute or API call, an invented statistic, a correct paper given the wrong journal, an economic formula with the wrong units. Show how you checked each one with a textbook, the official documentation or the original paper.

| # | AI tool | What we asked & what AI said | How we caught it | What was actually true (with source) | Error type | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| AI-01 | Select ▾ | type here | Select ▾ | type here | Select ▾ | Select ▾ |
| AI-02 | Select ▾ | type here | Select ▾ | type here | Select ▾ | Select ▾ |
| AI-03 | Select ▾ | type here | Select ▾ | type here | Select ▾ | Select ▾ |

5.2 Peer review: watch both assigned LinkedIn videos and score them (0–2 per criterion)

| Group reviewed | Problem | Physics | Business | Data | Clarity | Best moment (mm:ss) | One question for the viva |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Group 10: 7-DOF Surgical Robotic Assistant with Physiological Tremor Filtering for Precision Tasks | Select ▾ | Select ▾ | Select ▾ | Select ▾ | Select ▾ | type here | type here |
| Group 03: Autonomous Last-Mile Ground AED Delivery Robot for Sudden Cardiac Arrest | Select ▾ | Select ▾ | Select ▾ | Select ▾ | Select ▾ | type here | type here |

SECTION 6 · Individual Learning Record & Research Consent    unmarked · one row per member · answer honestly

These answers do not change your marks. They help improve the course and are used in anonymised research only if you consent in the sign-off below. Commit share: your commits ÷ the group's commits in your folder (git shortlog -sn -- <folder>).

| Member | Hours I spent (total) | My declared role matched my actual work (1 = not at all, 5 = fully) | I learned a skill outside my role (robotics ↔ business) | Scripts I mainly wrote (file names) | My commit share (%) | Share of my writing first drafted with AI | Wrong AI facts / code I personally caught |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Arnav Saurabh Surve (E064) | type here | Select ▾ | Select ▾ | type here | type here | Select ▾ | Select ▾ |
| Vihan Shripad Joshi (E070) | type here | Select ▾ | Select ▾ | type here | type here | Select ▾ | Select ▾ |
| Pratik Mangesh Gaikwad (E073) | type here | Select ▾ | Select ▾ | type here | type here | Select ▾ | Select ▾ |

| Member | Before this course I had used a physics simulator | The most valuable thing I learned (one line) | One thing I would change in this course (one line) |
| --- | --- | --- | --- |
| Arnav Saurabh Surve (E064) | Select ▾ | type here | type here |
| Vihan Shripad Joshi (E070) | Select ▾ | type here | type here |
| Pratik Mangesh Gaikwad (E073) | Select ▾ | type here | type here |

Track A: how this dossier becomes your 4-page IEEE paper

| I. Introduction | Section 1: title, verified baseline friction, governing standards |
| --- | --- |
| II. Related work | Section 1.1: verified sources only (5+ with DOIs) |
| III. System model | Section 2: MJCF model, FSM, equations, control law |
| IV. Techno-economics & evaluation | Sections 3–4: CSBS model, FMEA, Monte Carlo results, statistics |
| V. Discussion & limitations | Sim-to-real gap, assumptions in the business model, AI audit lessons |
| Target venues (titles sheet) | A: AIR · B: IEEE/RSJ IROS |

Individual Academic Sign-off    every member answers and signs

Answer each statement for yourself: (1) this work is original and every source and AI use is declared; (2) our model loads and our controller and business scripts run from the repository; (3) I understand our whole project and can explain my own part in the viva voce.

| Member (Roll) | (1) Original work | (2) Code runs | (3) Viva-ready | Signature | Optional research consent* |
| --- | --- | --- | --- | --- | --- |
| Arnav Saurabh Surve (E064) | Select ▾ | Select ▾ | Select ▾ |  | Select ▾ |
| Vihan Shripad Joshi (E070) | Select ▾ | Select ▾ | Select ▾ |  | Select ▾ |
| Pratik Mangesh Gaikwad (E073) | Select ▾ | Select ▾ | Select ▾ |  | Select ▾ |

*Optional: may Dr. Nanade use anonymised data from this dossier and repository in education research publications? Your answer has no effect on your marks, and you can withdraw until results are published.